White Paper — Mainstream Anti-Drone (C-UAS) Technologies

Prepared by Inphoeni Solutions
Author: Inphoeni Solutions— Solutions & White Papers
Date: 2025-11-11

[image: image1]
Executive summary

Unmanned Aerial Systems (UAS / “drones”) present a rapidly evolving threat across commercial, critical-infrastructure, and defense contexts. Modern counter-UAS (C-UAS) practice relies on layered detection + multi-mode mitigation, combining sensors (radar, RF, acoustic, EO/IR, ML) with effectors ranging from electronic warfare (jamming/spoofing) to physical capture and directed-energy weapons. This white paper summarizes the mainstream technologies in use today, their strengths & limits, operational considerations, and recommended layered architectures for site and perimeter protection. droneshield.com+1
[image: image2]
1. Problem framing: why layered C-UAS?

No single sensor or mitigation reliably handles the full spectrum of UAS threats (consumer quadcopters, fixed-wing long-endurance UAVs, autonomous swarms). Detection gaps (e.g., low-RCS small quadcopters), environmental effects (weather, RF clutter), and legal/regulatory limits on “hard-kill” countermeasures make multi-sensor, multi-effect layered systems the practical standard. Industry leaders emphasize sensor fusion and automated correlation to reduce false alarms and shorten response time. dedrone.com+1
[image: image3]
2. Detection technologies (sensing & identification)

A robust C-UAS begins with accurate detection, classification and tracking. Common sensor types:

2.1 Radar

Purpose: early detection and 3-D tracking (range, azimuth, altitude).

Strengths: weather-independent, can detect multiple targets and swarms.

Limits: small multirotors have low radar cross section (RCS) — short detection ranges; clutter in urban environments. Radar is commonly paired with other sensors for ID. dedrone.com+1
2.2 RF (Radio-frequency) sensing

Purpose: detect/control/telemetry signals between UAV and pilot — reveals presence, vendor/protocol, and sometimes approximate bearing.

Strengths: fast, effective against remote-controlled drones that use Wi-Fi/Proprietary RF links; can help locate the operator.

Limits: autonomous or GNSS-only drones, or encrypted/low-power links, can evade RF detection. RF is a primary input to many commercial systems. droneshield.com+1
2.3 Electro-Optical / Infrared (EO/IR) cameras

Purpose: visual confirmation, identification and classification (make/model, payload).

Strengths: high confidence ID, necessary for escalation to kinetic measures.

Limits: line-of-sight, day/night limitations without thermal, sensitive to weather and lighting. EO/IR often provides the visual “hand-over” after radar/RF cueing. droneshield.com
2.4 Acoustic sensors

Purpose: detect characteristic propeller/rotor noise signatures.

Strengths: low cost, covert, works when optical/RF fail (close range).

Limits: short ranges, false alarms from ambient noise. Good as part of sensor fusion. robinradar.com
2.5 ML / AI sensor fusion and classification

Purpose: combine radar/RF/EO/Acoustic to reduce false positives and classify threats to operator-actionable levels.

Strengths: automation, faster cueing of mitigations, improved classification over single sensors.

Limits: ML models require training for novel drone types and can be fooled by adversarial signals. droneshield.com
[image: image4]
3. Mitigation technologies (effectors)

Once a threat is detected and identified, operators choose mitigations consistent with legal rules of engagement and safety constraints. Mitigation types split into soft-kill (non-kinetic) and hard-kill (kinetic/directed energy).

3.1 Electronic Warfare — RF jamming (soft-kill)

What: transmit interfering RF energy on control and/or telemetry bands (2.4 GHz, 5.8 GHz, control, and video links) to break the link between UAS and operator.

Strengths: immediate neutralization of remote-piloted drones; widely fielded by commercial C-UAS.

Limits: illegal to use in many civil jurisdictions without authorization; may force some drones to fail-safe/return-to-home (which can be undesirable); ineffective against fully autonomous GNSS-based missions or military comms. droneshield.com+1
3.2 GNSS (GPS) jamming & spoofing (soft-kill / deception)

What: deny or feed false GNSS signals to disrupt navigation or lure a drone to a safe area.

Strengths: can neutralize long-range autonomous drones that rely on GNSS.

Limits: broad GNSS jamming can affect civil users/aircraft and raises heavy legal/regulatory issues; spoofing requires precision and carries risks of unpredictable drone behavior. Research identifies detection and mitigation of jamming/spoofing as a growing area. MDPI+1
3.3 Cyber takeover / protocol exploitation

What: exploit vendor-protocols to inject commands, take remote control, or crash drone software. Some commercial tools implement “command & control takeover” when vulnerabilities exist.

Strengths: potential for graceful recovery and evidence collection.

Limits: highly vendor-specific, legal/ethical concerns, requires deep protocol/firmware knowledge; not universally feasible. robinradar.com
3.4 Kinetic capture and physical intercept

Methods: net guns, projectile nets, interceptor drones (capture drones), trained birds of prey (rare), and shotgun/mortar approaches (military).

Strengths: physical removal of the UAS and payload, lower ambiguity when lawful to use.

Limits: risk of collateral damage, operator safety concerns, range limits, and rules of engagement. Good for localized, high-value point defense. robinradar.com
3.5 Directed-energy (lasers & high-power microwaves)

Lasers: high-energy lasers heat and damage airframe/components (sensors, propellers), enabling rapid, low-cost engagement at line-of-sight. Several national programs have matured into operational prototypes and fielded systems.

High-Power Microwaves (HPM): attempt to disrupt electronics at range to cause system failure.

Strengths: precise, instant engagements; cost-per-shot very low for lasers; scalable to engage swarms with high duty cycle.

Limits: power/size requirements, atmospheric attenuation (for lasers), safety/eye-safety/legal considerations, and large capital expenditure. Recent operational announcements (e.g., Iron Beam, DragonFire, naval laser demos) indicate increasing operational maturity. RTX+2Reuters+2
[image: image5]
4. Operational & legal considerations

Regulation: Many jurisdictions restrict RF/GNSS jamming and kinetic actions in civilian airspace — operators must coordinate with aviation authorities and obtain legal authorization.

Safety: Mitigations that cause a drone to fall (kinetic destruction) risk damage on the ground; mitigation choice should factor site footprint and collateral risk.

Attribution & forensics: RF capture and cyber takeover can provide evidence for prosecution; physical capture enables payload inspection. Sensor logs and video are essential.

Escalation model: Many vendors design systems with “observe → identify → warn → mitigate” workflows to ensure legal compliance and operator review before hard kills. NWS+1
[image: image6]
5. Typical architectures / recommended layered approach

Perimeter early warning layer: long-range radar + RF scanning to detect incoming threats.

Classification layer: EO/IR + acoustic + ML fusion to identify drone type and intent.

Decision / command layer: C2 console with rules engine for escalation and automatic alerts.

Mitigation layer: authorized mix of soft-kill (RF/GNSS jamming, cyber takeover) at stand-off distances; kinetic or directed-energy effectors where legally and operationally feasible.

Forensics & reporting: data logs, recording, and evidence packaging for law enforcement follow-up.
This layered approach reduces false alarms, confines interventions to necessary zones, and limits collateral risk. dedrone.com+1
[image: image7]
6. Case studies & market signals (short)

Commercial sensor fusion stacks (Dedrone, DroneShield): market leaders bundle radar, RF, EO/IR and ML to offer turnkey SaaS/C2 solutions for airports, prisons and critical infrastructure. These integrated offerings reflect industry move to sensor fusion rather than single-sensor deployments. dedrone.com+1
Directed-energy emergence: national programs (Israel’s Iron Beam, UK DragonFire trials, US naval laser demos) signal rapid maturation of laser systems for C-UAS roles — expected to be deployed increasingly for point defense against massed small threats. Operators should watch power, integration and legal frameworks closely. Reuters+1
[image: image8]
7. Practical selection guidance for buyers (Inphoeni Solutions lens)

When specifying a C-UAS solution, match capabilities to threat profile and environment:

Low-density urban site (e.g., event venue): focus on RF + EO/IR + acoustic fusion for short-range detection; non-destructive soft-kill (RF deny) preferred with law enforcement coordination. droneshield.com
Critical infrastructure (e.g., energy plant): invest in radar + RF long-range detection, EO/IR PTZ for ID, and authorized mitigation (cyber takeover or vetted kinetic / directed energy) with hardened fail-safes and containment planning. dedrone.com+1
Military / expeditionary use: integrate robust EW suites (broadband jamming, GNSS countermeasures), kinetic interceptors, and mobile directed-energy options; accept higher logistics and training overhead. es.ndu.edu
Checklist for procurement: detection range & minimum detectable RCS, integration & API for sensor fusion, false positive rate, escalation workflows, regulatory compliance support, evidence collection & chain of custody features, and vendor field support/training.

[image: image9]
8. Emerging trends to watch

Swarm defense: algorithms and effectors able to handle coordinated swarms (swarm detection by high-resolution radar + multi-beam lasers / HPM solutions).

AI adversary/defender arms race: adversarial ML and adaptive flight behaviors will require continual model retraining and threat-intelligence updates.

Smaller directed-energy systems: as lasers scale down in power/size while improving beam control, they will become viable for more fixed and mobile deployments.

Legal/regulatory evolution: expect stricter rules around jamming/spoofing and standardization of authorization processes (airspace safety, law enforcement protocols). Military Strategy Magazine+1
[image: image10]
9. Recommendations (actionable)

Adopt a layered, sensor-fusion architecture (radar + RF + EO/IR + acoustic + ML) as baseline for product offerings. dedrone.com+1
Offer modular mitigation packages: soft-kill (remote jamming/neutralization) as default commercial option; directed-energy / kinetic as optional modules where legally allowed. droneshield.com
Invest in operator UI & rules engine to minimize human error and ensure lawful escalation (observe → classify → warn → mitigate). NWS
Build a regulatory & forensics service to help customers obtain permissions and manage evidence chains; this is a competitive differentiator. droneshield.com
Monitor DEW (directed energy) developments and partnerships with laser providers for future product lines. 

